City of Pittsburgh

Sewer Facilities Planning Module Questionnaire

1) What was the previous permitted use for this property?

- Sports arena and assembly center for Duquesne University

2) What is the proposed use for the property?

- Use will remain the same, but the building footprint is being expanded due to University growth

3) How is green stormwater mitigation being integrated into the proposed project?

- The addition of street trees and an underground infiltration/detention tank system are being utilized for surface and rooftop runoff.  These best management practices (BMPs) provide detention of stormwater runoff.  The underground infiltration/detention tank system will help to reduce the 2-year runoff volume and peak flow rates.

4) Will the development result in a net positive or net negative change in stormwater flow?

- The proposed development will result in a net negative change in stormwater peak flow rates for the 1-, 2-, 5-, 10-, 25-, 50-, and 100-year design storms.  This standard is achieved by utilizing post construction stormwater management BMPs that include the addition of street trees and incorporating the underground infiltration/detention tank system.